ECE 382 Embedded Systems I

Word Operations (W) = 0

Byte Operations (B) = 1

Table 1. Source Addressing Modes (As)

| Address Mode | *As | Registers | Syntax | Operation |
| --- | --- | --- | --- | --- |
| Register | 00 | R0-R2, R4-R15 | Rn | Register Contents. |
| 0 | 00 | R3 | #0 | 0 Constant |
| Symbolic | 01 | R0 | addr | (PC+next word) points to operand. (x(PC)) |
| Indexed | 01 | R1, R4-R15 | x(Rn) | (Rn+x) points to operand. x is next code word. |
| Absolute | 01 | R2 | &addr | Next code word is the absolute address. (x(SR)) |
| +1 | 01 | R3 | #1 | +1 Constant |
| Indirect | 10 | R0-R1,R4-R15 | @Rn | Rn points to operand. |
| +4 | 10 | R2 | #4 | +4 Constant |
| +2 | 10 | R3 | #2 | +2 Constant |
| Immediate | 11 | R0 | #N | Next word is the constant N. (@PC+) |
| Indirect auto-inc | 11 | R1,R4-R15 | @Rn+ | Rn points to operand, Rn is incremented (1 or 2). |
| +8 | 11 | R2 | #8 | +8 Constant |
| -1 | 11 | R3 | #-1 | -1 Constant |

*Bits 4 and 5 in Single (Table 3) and Double (Table 5) Operand Instructions

Table 2. Destination Addressing Modes (Ad)

| Address Mode | *Ad | Registers | Syntax | Operation |
| --- | --- | --- | --- | --- |
| Register | 0 | R0-R2, R4-R15 | Rn | Register Contents. |
| 0 | 0 | R3 | #0 | Bit bucket |
| Symbolic | 1 | R0 | addr | (PC+next word) points to operand. (x(PC)) |
| Indexed | 1 | R1, R4-R15 | x(Rn) | (Rn+x) points to operand. x is next code word. |
| Absolute | 1 | R2 | &addr | Next code word is the absolute address. (x(SR)) |

*Bit 7 in (Table 5) Operand Instructions

Table 3. Single Operand Instructions

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9-bit Opcode |  |  |  |  |  |  |  |  | b/w | As |  | D/S Register |  |  |  |

| Mnemonic | Opcode |  |  |  |  |  |  |  |  | V | N | Z | C | Operation | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | • | • | • | • | CMSB…LSBC | Roll dst right through C |
| SWPB | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | – | – | – | – | Swap bytes | Swap bytes |
| RRA | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | • | • | • | MSBMSB…LSBC | Roll destination right |
| SXT | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | • | • | z | bit 7bit 8…bit 15 | Sign extend destination |
| PUSH | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | – | – | – | – | SP-2SP, src@SP | Push source on stack |
| CALL | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | – | – | – | – | SP-2SP, PC+2@SP, dstPC | Subroutine call |
| RETI | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | • | • | • | • | @SP+SR, @SP+PC | Return from interrupt |

Table 4. Jump Instructions

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6-bit Opcode |  |  |  |  |  | 10-bit, 2’s complement PC Offset |  |  |  |  |  |  |  |  |  |

| Mnemonic | Opcode |  |  |  |  |  | V | N | Z | C | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JNZ/JNE | 0 | 0 | 1 | 0 | 0 | 0 | – | – | 0 | – | Jump if not equal |
| JZ/JEQ | 0 | 0 | 1 | 0 | 0 | 1 | – | – | 1 | – | Jump if equal |
| JNC/JLO | 0 | 0 | 1 | 0 | 1 | 0 | – | – | – | 0 | Jump if carry flag equal to zero |
| JC/JHS | 0 | 0 | 1 | 0 | 1 | 1 | – | – | – | 1 | Jump if carry flag equal to one |
| JN | 0 | 0 | 1 | 1 | 0 | 0 | – | 1 | – | – | Jump if negative (N = 1) |
| JGE | 0 | 0 | 1 | 1 | 0 | 1 | • | • | – | – | Jump if greater than or equal (N = V) |
| JL | 0 | 0 | 1 | 1 | 1 | 0 | • | • | – | – | Jump if lower (N V) |
| JMP | 0 | 0 | 1 | 1 | 1 | 1 | – | – | – | – | Unconditional jump |

Table 5. Double Operand Instructions

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-bit Opcode |  |  |  | Source Register |  |  |  | Ad | b/w | As |  | Destination Register |  |  |  |

| Mnemonic | Opcode |  |  |  | V | N | Z | C | Operation | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV | 0 | 1 | 0 | 0 | – | – | – | – | srcdst | Move source to destination |
| ADD | 0 | 1 | 0 | 1 | • | • | • | • | src+dstdst | Add source to destination |
| ADDC | 0 | 1 | 1 | 0 | • | • | • | • | src+dst+Cdst | Add src and C to dst |
| SUBC | 0 | 1 | 1 | 1 | • | • | • | • | dst+.not.src+Cdst | Subtract src and NOT C from dst |
| SUB | 1 | 0 | 0 | 0 | • | • | • | • | dst+.not.src+1dst | Subtract source from destination |
| CMP | 1 | 0 | 0 | 1 | • | • | • | • | dst-src | Compare source to destination |
| DADD | 1 | 0 | 1 | 0 | • | • | • | • | src+dst+Cdst(dec) | Decimal add src and C to dst |
| BIT | 1 | 0 | 1 | 1 | 0 | • | • | z | src.and.dst | Test bits in destination |
| BIC | 1 | 1 | 0 | 0 | – | – | – | – | .not.src.and.dstdst | Clear bits in destination |
| BIS | 1 | 1 | 0 | 1 | – | – | – | – | src.or.dstdst | Set bits in destination |
| XOR | 1 | 1 | 1 | 0 | • | • | • | z | src.xor.dstdst | XOR source with destination |
| AND | 1 | 1 | 1 | 1 | 0 | • | • | z | src.and.dstdst | AND source with destination |

Status Register: • = bit affected,  – =  bit not affected, 0 = cleared, 1 = set, z = same as Z

Table 6. Source Operands Using Status (R2) and Constant Generator (R3) Registers

| Register | As | Addr Mode | Syntax | Constant | Remarks |
| --- | --- | --- | --- | --- | --- |
| R2 | 00 | Register | – | – | Register mode |
| R2 | 01 | x(R2) | addr | (0) | Absolute address mode, next word contains address |
| R2 | 10 | @R2 | #4 | 0x0004 | +4 |
| R2 | 11 | @R2+ | #8 | 0x0008 | +8 |
| R3 | 00 | R3 | #0 | 0x0000 | 0 |
| R3 | 01 | x(R3) | #1 | 0x0001 | +1, No extension word |
| R3 | 10 | @R3 | #2 | 0x0002 | +2 |
| R3 | 11 | @R3+ | #-1 | 0xFFFF | -1 |

Mnemonic		Description	Operation	V	N	Z	C

ADC(.B) 	dst 	Add C to destination 	dst + C → dst 	*	* 	* 	*

ADD(.B) 	src,dst 	Add source to destination 	src + dst → dst 	* 	* 	* 	*

ADDC(.B) 	src,dst 	Add source and C to destination 	src + dst + C → dst 	* 	* 	* 	*

AND(.B) 	src,dst 	AND source and destination	src .and. dst → dst 	0 	* 	* 	*

BIC(.B) 	src,dst 	Clear bits in destination 	.not.src .and. dst → dst 	- 	- 	-	-

BIS(.B) 	src,dst 	Set bits in destination 	src .or. dst → dst 	- 	- 	- 	-

BIT(.B) 	src,dst 	Test bits in destination 	src .and. dst 	0 	* 	* 	*

BR 	dst 	Branch to destination 	dst → PC 	- 	- 	- 	-

CALL 	dst 	Call destination 	PC+2 → stack, dst → PC 	- 	- 	- 	-

CLR(.B) 	dst 	Clear destination 	0 → dst 	- 	- 	- 	-

CLRC		Clear C 	0 → C 	- 	- 	- 	0

CLRN		Clear N 	0 → N	- 	0	 - 	-

CLRZ		Clear Z 	0 → Z 	- 	- 	0 	-

CMP(.B) 	src,dst 	Compare source and destination 	dst - src 	* 	* 	* 	*

DADC(.B) 	dst 	Add C decimally to destination 	dst + C → dst (decimally) 	* 	* 	* 	*

DADD(.B) 	src,dst 	Add source and C decimally to dst 	src + dst + C → dst (decimally) 	* 	* 	* 	*

DEC(.B) 	dst 	Decrement destination 	dst - 1 → dst 	* 	* 	* 	*

DECD(.B) 	dst 	Double-decrement destination 	dst - 2 → dst 	* 	* 	* 	*

DINT  		Disable interrupts 	0 → GIE 	- 	- 	- 	-

EINT  		Enable interrupts 	1 → GIE 	- 	- 	- 	-

INC(.B) 	dst 	Increment destination 	dst +1 → dst 	* 	* 	* 	*

INCD(.B) 	dst 	Double-increment destination 	dst+2 → dst 	* 	* 	* 	*

INV(.B)  	dst 	Invert destination 	.not.dst → dst 	* 	* 	* 	*

JC/JHS 	label 	Jump if C set/Jump if higher or same 		- 	- 	- 	-

JEQ/JZ 	label 	Jump if equal/Jump if Z set 		- 	- 	- 	-

JGE 	label 	Jump if greater or equal 		- 	- 	- 	-

JL 	label 	Jump if less 		- 	- 	- 	-

JMP 	label 	Jump PC + 2 × offset → PC		- 	- 	- 	-

JN 	label 	Jump if N set 		- 	- 	- 	-

JNC/JLO 	label 	Jump if C not set/Jump if lower 		- 	- 	- 	-

JNE/JNZ 	label 	Jump if not equal/Jump if Z not set 		- 	- 	- 	-

MOV(.B) 	src,dst 	Move source to destination src → dst 		- 	- 	- 	-

NOP		No operation 		- 	- 	- 	-

POP(.B) 	dst 	Pop item from stack to destination 	@SP → dst, SP+2 → SP	- 	- 	- 	-

PUSH(.B) 	src 	Push source onto stack 	SP - 2 → SP, src → @SP	- 	- 	- 	-

RET		Return from subroutine 	@SP → PC, SP + 2 → SP 	- 	- 	- 	-

RETI 		Return from interrupt 		* 	* 	* 	*

RLA(.B) 	dst 	Rotate left arithmetically 		* 	* 	* 	*

RLC(.B) 	dst 	Rotate left through C 		* 	* 	* 	*

RRA(.B) 	dst 	Rotate right arithmetically 		0 	* 	* 	*

RRC(.B) 	dst 	Rotate right through C 		* 	* 	* 	*

SBC(.B) 	dst 	Subtract not(C) from destination 	dst + 0FFFFh + C → dst 	* 	* 	* 	*

SETC		Set C 	1 → C 	- 	- 	- 	1

SETN		Set N 	1 → N 	- 	1	 - 	-

SETZ		Set Z 	1 → Z 	- 	- 	1 	-

SUB(.B) 	src,dst 	Subtract source from destination 	dst + .not.src + 1 → dst 	* 	* 	* 	*

SUBC(.B) 	src,dst 	Subtract source and not(C) from 	dst dst + .not.src + C → dst 	* 	* 	* 	*

SWPB 	dst 	Swap bytes 		- 	- 	- 	-

SXT 	dst 	Extend sign 		0	* 	* 	*

TST(.B) 	dst 	Test destination 	dst + 0FFFFh + 1 	0 	* 	* 	1

XOR(.B) 	src,dst 	Exclusive OR source and destination 	src .xor. dst → dst 	* 	* 	* 	*

Legend: 	* 	= 	Status bit cleared or set on results 	- 	= 	Status bit not affected

0 	= 	Status bit always cleared 	1 	= 	Status bit always set

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | Opcode |  |  | W=0/B=1 | As |  | Dest Reg |  |  |  |
| 0 | 0 | 1 | Condition |  |  | PC Offset (10-Bit) |  |  |  |  |  |  |  |  |  |
| Opcode |  |  |  | Source Reg |  |  |  | Ad | W=0/B=1 | As |  | Dest Reg |  |  |  |

03FFh

LED1 is P1.0

LED2 is P1.6